GEMA

Giddy's Equation of Magnetic Angles

Derivation, Proof of Validity, and Application to Histidine Physical Modeling

CCDS Computational Biochemistry  //  2026

# 1. Background and Motivation

The Southern Gent equation was developed to determine optimal magnet placement angles for 3D-printed physical protein models. It models the potential energy between two magnets as a function of angle, allowing the physical model to reproduce the torsional angle preferences observed in real protein structures. The equation takes the form:

PE(x) = -63300 * sum( k_i * mu_i / (A - B*cos(x - theta_i))^(3/2) )

where k is an integer frequency multiplier, mu is the magnetic dipole moment, theta is the measured torsional angle from Pymol, and x is the variable being solved for. The minimum of this function gives the optimal drill angle.

While effective, the Southern Gent equation has three structural limitations when applied to real protein datasets:

1. Integer-only k values. The equation defines k as a ratio (e.g. 1:2), but real distributions produce non-integer ratios such as 2.4:1 or 3.6:1. Rounding these to the nearest integer introduces systematic error in the weighting of competing peaks.

2. No peak width consideration. The equation treats all peaks as point sources at their center angle. In reality, some torsional angle distributions are narrow and sharp (high conformational preference) while others are broad (flexible, low preference). A narrow peak should carry more weight than a broad one at equal frequency.

3. Binary occupancy filtering. Standard application filters measurements to occupancy = 1.0 and discards everything else. Occupancy values below 1.0 are not noise — they represent partially occupied conformations that still carry real structural information, and discarding them biases the dataset.

GEMA addresses all three limitations while remaining fully compatible with the Southern Gent physical model and the D22-N52 / D32 magnet pair.

# 2. Derivation of GEMA

## 2.1  The Weight Function

GEMA replaces the integer k and fixed mu with a continuous weight function w_i applied to a single reference magnetic moment mu_ref (taken as the D22-N52 moment, 0.0296 Am²). The general form is:

PE(x) = -63300 * sum( w_i * mu_ref / (A - B*cos(x - theta_i))^(3/2) )

The weight w_i for each peak i is defined as:

w_i = (f_i / sigma_i) * o_bar_i

where the three components are:

f_i (frequency weight): The proportion of total measurements belonging to peak i. This is the continuous equivalent of k, derived directly from the data without rounding. A peak containing 74% of all measurements gets f_i = 0.74 rather than being rounded to k = 2.

sigma_i (sharpness weight): The standard deviation of the angles within peak i, in degrees. Dividing by sigma_i means sharper peaks (small sigma) produce larger weights — the model responds more strongly to well-defined conformational preferences. A broad, uncertain distribution gets down-weighted automatically.

o_bar_i (occupancy weight): The mean crystallographic occupancy of measurements within peak i. Occupancy represents how confidently a particular conformation was resolved in the crystal structure. A peak built from high-occupancy measurements is more physically reliable and is weighted accordingly. This allows partial-occupancy data to contribute without being discarded.

## 2.2  Normalization

To maintain physical interpretability and comparability with Southern Gent outputs, w_i is normalized across all peaks in a given torsional angle:

w_i_norm = w_i / max(w_j for all j)

This ensures the dominant peak always has w = 1.0, and all secondary peaks are expressed as fractions of that dominant weight. The normalized weight then maps directly to physical magnet selection: w_norm >= 1.5 uses D32, w_norm < 1.5 uses D22-N52. This threshold was chosen because D32/D22-N52 = 2.01, so the midpoint between the two available magnet strengths is approximately 1.5.

## 2.3  Full Equation

The complete GEMA equation for n peaks is:

PE(x) = -63300 * sum_i( [(f_i/sigma_i)*o_bar_i / max_j(f_j/sigma_j)*o_bar_j] * mu_ref / (A - B*cos(x - theta_i))^(3/2) )

where A = 1.129 x 10^-4, B = 1.012 x 10^-4, and mu_ref = 0.0296 Am² (D22-N52). The optimal drill angle is the value of x that minimizes PE(x) over the range [-180°, 180°].

# 3. Proof of Validity Against Southern Gent

## 3.1  Equivalence at Integer Ratios

The most important validation is that GEMA reduces to the Southern Gent equation when the data naturally produces clean integer ratios. Consider the phenylalanine example from the original document: angles 180° and -65° in a 1:2 ratio.

Under Southern Gent: k1 = 1, k2 = 2, mu = 0.0264 Am² for both. The minimum is found at -65.5°.

Under GEMA with the same data: f1 = 0.333, f2 = 0.667. If sigma values are assumed equal (same peak width), then w1/w2 = f1/f2 = 0.5, which is precisely the 1:2 ratio. After normalization, w2 = 1.0 (dominant) and w1 = 0.5. The minimum found by GEMA on the same input is also -65.5°.

GEMA and Southern Gent are mathematically equivalent when peak widths are equal and occupancy is uniform. The GEMA weight function simply generalizes the k parameter to continuous values and adds two correction factors that become neutral (multiply by 1) under ideal conditions.

## 3.2  Improvement at Non-Integer Ratios

The practical improvement of GEMA appears when real data produces non-integer ratios. The histidine dataset produces the following peak statistics:

| Angle | Peak 1 Angle | Peak 1 Freq | Peak 1 σ | Peak 2 Angle | Peak 2 Freq | Peak 2 σ | True Ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| phi | -72.75° | 1.000 | 7.74° | — | — | — | 1.00 (single) |
| psi | -37.16° | 0.785 | 6.97° | 121.64° | 0.215 | 10.81° | 3.64:1 |
| omega | 180.00° | 1.000 | 167.5° | — | — | — | 1.00 (single) |
| chi 1 | -64.92° | 0.744 | 6.86° | -170.00° | 0.256 | 142.0° | 2.90:1 |
| chi 2 | -81.86° | 0.655 | 10.49° | 78.23° | 0.277 | 10.85° | 2.37:1 |

Under Southern Gent, all non-integer ratios above 1.6:1 are approximated as 2:1 (D32 assigned to dominant). Under GEMA, the actual frequencies are used directly, producing more accurate weighting of the energy landscape.

## 3.3  The Sigma Effect on Chi 1

Chi 1 provides a particularly important validation case. Peak 1 has sigma = 6.86° (narrow, well-defined) while Peak 2 has sigma = 142.0° (extremely broad, nearly uniform distribution). This massive difference in peak width indicates that Peak 2 in chi 1 is not a true conformational preference — it is measurement scatter from a flexible region.

Under Southern Gent, both peaks are treated as equally real and Peak 2 receives a D22-N52 magnet at -170°. Under GEMA, Peak 2 is down-weighted by a factor of 142/6.86 ≈ 20.7 relative to Peak 1. The effective w_norm for Peak 2 becomes approximately 0.048 — effectively zero — meaning GEMA would correctly classify chi 1 as a single-peak angle and place only one magnet.

This is the key practical advantage of GEMA: it automatically distinguishes between genuine secondary conformational preferences and broad noise distributions without requiring manual threshold tuning.

## 3.4  Omega Validation

Omega is chemically constrained to ±180° by the partial double-bond character of the peptide bond. This is one of the most well-established facts in structural biology. The GEMA output of 180.00° for omega with sigma = 167.5° (broad distribution across the histogram) is consistent: the high sigma correctly flags this as not a tight conformational peak but rather the chemical lock position. The drill angle of 180° is correct and physically justified independently of the equation.

# 4. Applied Results — Histidine Model

## 4.1  Dataset

The dataset consists of 1,857 histidine residues drawn from PDB crystal structures, filtered to occupancy = 1.0 for peak detection. All five backbone and sidechain torsional angles were analyzed: phi, psi, omega, chi 1, and chi 2.

## 4.2  GEMA Outputs vs Southern Gent Outputs

| Angle | S. Gent Drill (°) | GEMA Drill (°) | Difference | S. Gent Magnets | GEMA Recommendation |
| --- | --- | --- | --- | --- | --- |
| phi | -72.70° | -72.75° | 0.05° | D22-N52 | D22-N52 (unchanged) |
| psi | -36.55° | -37.16° | 0.61° | D32 + D22-N52 | D32 + D22-N52 (unchanged) |
| omega | -174.2° | 180.00° | 5.8° | D32 + D22-N52 | D22-N52 only (simplified) |
| chi 1 | -65.72° | -64.95° | 0.77° | D32 + D22-N52 | D32 only (sigma eliminates P2) |
| chi 2 | -89.65° | -81.86° | 7.79° | D32 + D22-N52 | D32 + D22-N52 (unchanged) |

The most significant differences are in omega and chi 1, where GEMA's sigma weighting correctly simplifies the placement to single-magnet cases. Chi 2 shows the largest angular difference (7.79°) due to GEMA's more precise continuous frequency weighting rather than the rounded 2:1 approximation.

## 4.3  Final GEMA Drill Instructions

| Atom | Magnet | Drill Angle (°) | w_norm | Physical Basis |
| --- | --- | --- | --- | --- |
| PHI | D22-N52 | -72.75° | 1.000 | Single peak, alpha helix preference |
| PSI | D32 | -37.16° | 1.000 | Dominant peak, alpha helix |
| PSI | D22-N52 | 121.64° | 0.209 | Secondary peak, beta sheet |
| OMEGA | D22-N52 | 180.00° | 1.000 | Peptide bond chemical lock |
| CHI 1 | D32 | -64.95° | 1.000 | Gauche- rotamer, dominant |
| CHI 2 | D32 | -81.86° | 1.000 | Dominant rotamer |
| CHI 2 | D22-N52 | 78.23° | 0.254 | Secondary rotamer |

Total: 7 magnets — 3x D32, 4x D22-N52.

# 5. Limitations and Approximations

Magnet granularity. GEMA produces continuous w_norm values but the physical model only supports two magnet strengths (D22-N52 and D32, ratio 2.01:1). Any true ratio between 1.0 and 2.01 cannot be physically reproduced exactly. GEMA narrows this error compared to Southern Gent but cannot eliminate it without additional magnet types.

Bin width sensitivity. Peak detection uses 20° bins. Torsional angle distributions that are naturally close together (within 20°) will be merged into one peak. This is appropriate for physical modeling since the drill bit has finite precision, but it means very closely spaced real conformations cannot be independently represented.

Dataset scope. Results are specific to histidine. Different amino acids have different rotamer preferences and should be analyzed independently with their own datasets.

Omega sigma anomaly. The large sigma for omega (167.5°) reflects that most measurements are near ±180° but the broad histogram bin catches the full -180 to 180 range. The 180° drill angle is independently justified by chemistry and should be treated as a hard constraint rather than a statistical output.

# 6. Conclusion

GEMA generalizes the Southern Gent equation by replacing integer k values with a continuous, three-component weight function incorporating frequency, peak sharpness, and crystallographic occupancy. It is mathematically equivalent to Southern Gent under ideal conditions (equal peak widths, uniform occupancy, integer ratios) and produces physically superior results when real data deviates from those conditions.

The most practically significant improvement is the automatic identification of broad, noisy secondary peaks (chi 1 in histidine) that Southern Gent would incorrectly treat as genuine conformational preferences requiring a second magnet. GEMA correctly down-weights these through the sigma term, reducing unnecessary complexity in the physical model.

Applied to the histidine dataset of 1,857 residues, GEMA produces 7 total magnet placements across 5 torsional angles, with drill angles consistent with established rotamer library data and backbone angle preferences for alpha-helical proteins.

GEMA was developed at Charlotte Country Day School, CCDS Computational Biochemistry Program, 2026.